- PC104-3610/3620

PC/104 Ethernet Module

PC/104 Dual Ethernet Module

User’s Manual

(Version 2.1)

- Copyright Notice

This document is copyrighted, 2000 by NAGASAKI International Corp. All rights are reserved. The information in the manual is subject to change without notice in order to improving products.

No part of this manual may be reproduced, copied, translated or transmitted in any form or by any means without the prior written permission of the manufacturer.

NAGASAKI International Corp. assumes no responsibility for any inaccuracies that may be contained in this document. NAGASAKI International Corp. makes no commitment to update or to keep current the information contained in this manual.

Copyright 2000 by NAGASAKI International Corp.

- All rights reserved. Ver.2.1 2000,

- Printed in Taiwan

- Trademarks Acknowledgments

All brand names and trademarks are the properties and registered brands of their respective owners.

# T a b l e  o f  C o n t e n t s

Chapter 0	Packing List		1

Chapter 1	Specifications		1

Chapter 2	Jumper Settings	          4

Chapter 3	Connectors		          7

Chapter 4	Netwaork Interface	7

Warranty					8

# C h a p t e r  0

Packing List

| Function | Function | Package |
| --- | --- | --- |
| PC104-3610 | Ethernet Module | - ICOP-2800 PC/104 Ethernet Module - Utility and Drivers Diskette x 1 - BNC T-type connector |
| PC104-3620 | Dual Ethernet Module | - PC104-3620PC/104 Dual Ethernet Module - Utility and Drivers Diskette x 1 |

# C h a p t e r  1

Specifications

| Features | PC104-3610 | PC104-3620 |
| --- | --- | --- |
| Chipset | Realtek RTL8019AS |  |
| Chipset Number | 1 | 2 |
| Bus Interface | PC/104 standard compliant |  |
| Connector | BNC, RJ-45 | RJ-45 X 2 |
| BIOS Socket | 1 | 0 |
| Power Requirement | +5V @120mA | +5V @120mA |
| Board Weight | 100g | 80g |
| Board Size | 96mm X 90 mm |  |
| Operating Temperature | 0 ~ +60°C |  |

## Description

The RTL8019AS integrates the RTL8019A and 16K-byte SRAM in a single chip. It is designed not only to provide more friendly functions but also to save the effort of SRAM source and inventory. Most of the design's specification inherit from RTL8019A.

Compliant to Ethernet II and IEEE802.3 10Base5, 10Base2, 10BaseT

Supports UTP, AUI & BNC auto-detect

Software compatible with NE2000

Supports PnP auto detect mode

Supports Full-Duplex Ethernet function to double channel bandwidth

Supports three level power down modes: sleep, power down with internal clock running clock, power down with internal halted.

Built-in data prefetch function to improve performance

Support 8 IRQ lines

Supports IO address fully decode mode

Supports 16K, 32K, 64K and 16K-page mode access to BROM (up to 256 pages with 16K bytes/page), and Flash memory read/write

Improves the blinking problem of CRS LED when the cable is not connedted

Software Support/Application Drivers:

NDIS2 (DOS, OS/2,Lantastic, WFW3.1, etc.)

NDIS3, NDIS4, NDIS5 for WIN95, 98, NT3.51, 4.0, 5.0, WFW3.11

Netware 16-bit ODI driver for DOS, OS/2 and 32-bit ODI driver for Netware 3.x, 4.x, 5.0 Server

Packet driver for UNIX Client

SCO UNIX driver

Linux driver

Interface: PC/104 standard compliant

Connector:

BNC connector for 10Base2

RJ-45 connector for 10BaseT

Power Requirements: single 5V @ 120 mA

Dimensions: 90 (L) x 96 (W) mm.

Weight: 100 g (PC104-3610), 80 g (PC104-3620)

Operating Temperature: 0 ~ +60 °C

## Product Photo

PC104-3610

PC104-3620

# C h a p t e r  2

Jumper Setting

PC104-3610

JP: hardware/software mode selection

close: Jumper mode

open:  software mode

PnP: plug and play mode

close: enable plug and play mode

open: disable plug and play mode

BS#	BROM size and memory base address

| BS4 | BS3 | BS2 | BS1 | BS0 | BROM base and size |
| --- | --- | --- | --- | --- | --- |
| open | open | * | * | * | Disabled * |
| open | close | open | open | open | C000H, 32K |
| open | close | open | open | close | C800H, 32K |
| open | close | open | close | open | D000H, 32K |
| open | close | open | close | close | D800H, 32K |
| open | close | close | open | open | C000H, 64K |
| open | close | close | open | close | D000H, 64K |
| close | open | open | open | open | C000H, 16K |
| close | open | open | open | close | C400H, 16K |
| close | open | open | close | open | C800H, 16K |
| close | open | open | close | close | CC00H, 16K |
| close | open | close | open | open | D000H, 16K |
| close | open | close | open | close | D400H, 16K |
| close | open | close | close | open | D800H, 16K |
| close | open | close | close | close | DC00H, 16K |
| close | close | open | open | open | C000H, PAGE |
| close | close | open | open | close | C400H, PAGE |
| close | close | open | close | open | C800H, PAGE |
| close | close | open | close | close | CC00H, PAGE |
| close | close | close | open | open | D000H, PAGE |
| close | close | close | open | close | D400H, PAGE |
| close | close | close | close | open | D800H, PAGE |
| close | close | close | close | close | DC00H, PAGE |

IOS#	Select I/O base address

| IOS3 | IOS2 | IOS1 | IOS0 | I/O Base |
| --- | --- | --- | --- | --- |
| open | open | open | open | 300H |
| open | open | open | close | 320H * |
| open | open | close | open | 340H |
| open | open | close | close | 360H |
| close | open | open | open | 380H |
| close | open | open | close | 3A0H |
| close | open | close | open | 3C0H |
| close | open | close | close | 3E0H |
| open | close | open | open | 200H |
| open | close | open | close | 220H |
| open | close | close | open | 240H |
| open | close | close | close | 260H |
| close | close | open | open | 280H |
| close | close | open | close | 2A0H |
| close | close | close | open | 2C0H |
| close | close | close | close | 2E0H |

PL#	Select network medium types

| PL1 | PL0 | Medium Type |
| --- | --- | --- |
| open | open | TP/CX auto-detect (10BaseT link test is enabled) |
| open | close | 10BaseT with link test disabled |
| close | open | 10BaseT (RJ-45) |
| close | close | 10Base2 (BNC) * |

IRQ#	IRQ Select

| #2 | #1 | #0 | Interrupt Line | Assigned ISA IRQ |
| --- | --- | --- | --- | --- |
| open | open | open | INT0 | IRQ2/9 |
| open | open | close | INT1 | IRQ3 |
| open | close | open | INT2 | IRQ4 |
| open | close | close | INT3 | IRQ5 * |
| close | open | open | INT4 | IRQ10 |
| close | open | close | INT5 | IRQ11 |
| close | close | open | INT6 | IRQ12 |
| close | close | close | INT7 | IRQ15 |

PC104-3620(J3): NET# enable/disable

|  | NET1 | NET2 |
| --- | --- | --- |
| Enable * | 1-3 close | 2-4 close |
| Disable | 3-5 close | 4-6 close |

# C h a p t e r  3

Connectors

PC104-3610

CONT1 	64-pin PC/104 bus

CONT2 	40-pin PC/104 bus

CONT3 	BNC connector

CONT6 	RJ-45 connector

PC104-3620

J1 	64-pin PC/104 bus

J2 	40-pin PC/104 bus

TP1 	NET1 RJ-45 connector

TP2 	NET1 RJ-45 connector

# C h a p t e r  4

Network Interface

The Realtek RTL-8019AS 10Mbps Ethernet controller board supports both 10BASE-T 'RJ-45' and Coax 10Base-2 ‘BNC’ connectors, and allows direct connection to your 10Mbps Ethernet based Local Area Network for full interaction with local servers, wide area networks such as the Internet.

I/O and IRQ settings can be done by software with the supplied utility software, or it can be set for Plug and Play compatibility. The controller supports: Full-Duplex Ethernet function to double channel bandwidth, auto media detection.

# Warranty

This product is warranted to be in good working order for a period of one year from the date of purchase. Should this product fail to be in good working order at any time during this period, we will, at our option, replace or repair it at no additional charge except as set forth in the following terms. This warranty does not apply to products damaged by misuse, modifications, accident or disaster. Vendor assumes no liability for any damages, lost profits, lost savings or any other incidental or consequential damage resulting from the use, misuse of, or inability to use this product. Vendor will not be liable for any claim made by any other related party. Return authorization must be obtained from the vendor before returned merchandise will be accepted. Authorization can be obtained by calling or faxing the vendor and requesting a Return Merchandise Authorization (RMA) number. Returned goods should always be accompanied by a clear problem description.