|  | Task Hazard Analysis (THA) Worksheet (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) | Click For Word Doc |
| --- | --- | --- |

| Author: | Paul Brindza |  |  |  |  | Date: |  | 10/11/16 |  |  | Task #: If applicable | OSP 63388 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Complete all information. Use as many sheets as necessary |  |  |  |  |  |  |  |  |  |  |  |  |
| Task Title: |  | Q2Q3D Acceptance Testing |  |  |  |  |  |  | Task Location: | Hall C SHMS |  |  |
| Division: |  | Physics |  |  | Department: |  | Hall C |  |  | Frequency of use: |  | 36 months |
| Lead Worker: |  |  | Paul Brindza |  |  |  |  |  |  |  |  |  |
| Mitigation already in place: Standard Protecting Measures Work Control Documents |  |  |  | OSP 63388 |  |  |  |  |  |  |  |  |

| Sequence of Task Steps | Task Steps/Potential Hazards | Consequence Level | Probability Level | Risk Code (before mitigation) | Proposed Mitigation (Required for Risk Code >2) | Safety Procedures/ Practices/Controls/Training | Risk Code (after mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Magnetic field | low | low | 1 | High Magnetic fields not accessible, external low magnetic fields to be measured, Posting Magnetic fields greater than 50 Gauss possible, Access control | NA | NA |
|  | Electrical voltages 270 V high currents 3500 Amps Hipot testing 500 V | Low | Med | 2 | NEMA enclosures, guards, rated cables, Hipot testing, equipment has earth ground | Elec worker training, OSP 63388. Q2Q3D ERR Review | 1 |
|  | Pressure, 6 Atm gauge | Low | Med | 2 | ASME vessel rating, ASME relief devices, Analysis | OSP 63388, Q2Q3D ERR Review, JLAB pressure system design authority review, pressure test to 115% MAWP | 1 |
|  | Cryogenic, LHE, LN2, ODH | Low | Low | 1 | Cryogens not accessible, relief devices not accessible | OSP 63388, ODH training, Hall C ODH Analysis | Na |
|  | Magnet cool down | Low | Low | 1 | Cool down controlled and interlocked to limit magnet delta T internal to < 50 Kelvin to avoid thermal stress | OSP63388, Q2Q3D ERR review | NA |
|  | Magnet Quench | Low | Med | 2 | Active quench detection and protection prevent magnet overheating by fast energy removal, Magnet design precludes excessive heating by design | OSP 63388, Q2Q3D ERR review | 1 |

| Highest Risk Code before Mitigation: | 2 | Highest Risk Code after Mitigation: | 1 |
| --- | --- | --- | --- |

When completed, if the analysis indicates that the Risk Code before mitigation for any steps is “medium” or higher (RC≥3), then a formal Work Control Document (WCD) is developed for the task.  Attach this completed Task Hazard Analysis Worksheet.  Have the package reviewed and approved prior to beginning work.  (See ES&H Manual Chapter 3310 Operational Safety Procedure Program.)

Form Revision Summary Periodic Review – 08/13/15 – No changes per TPOC Revision 0.1 – 06/19/12 - Triennial Review. Update to format. Revision 0.0 – 10/05/09 – Written to document current laboratory operational procedure. ISSUING AUTHORITY; TECHNICAL POINT-OF-CONTACT; APPROVAL DATE; REVIEW DATE; REV.. ESH&Q Division; Harry Fanning; 08/13/15; 08/13/18; 0.1. This document is controlled as an on line file. It may be printed but the print copy is not a controlled document. It is the user’s responsibility to ensure that the document is the same revision as the current on line file. This copy was printed on 7/30/2026.